Asbestos Policy

| Document Title | Asbestos Policy |
| --- | --- |
| Location | Head office - Broadacres Housing Association, Broadacres House, Mount View, Standard Way, Northallerton, North Yorkshire, DL6 2YD |
| Version | 2.0 |
| Level | Asset and Development Committee |
| Owner / Position | Andy Powell - Customer Experience Director |
| Reviewed By | Tony Warbrook |
| Approved By | A&D Committee |
| Approval Date |  |
| Review Period | 2 yearly |
| Next Review | November 2026 |

| Version | Date | Comments |
| --- | --- | --- |
| 2 | 10/2019 | Removed requirement for 30 day reporting. |
| 3 | November 2024 | - 11.1 Qualification no longer available 13.3 Clarity on UKAS audit process. Minor amendments to clarify approach. |

Contents

- Introduction	2

- Scope	3

- Regulatory Standards, Legislation and Codes of Practice	3

- Additional Legislation	4

- Obligations	5

- Statement of Intent	5

- Compliance Risk Assessment and Inspection Testing Programmes	7

- Compliance Follow up Work	8

- Record Keeping	9

- Roles and Responsibilities	9

- Competent Persons	10

- Training	10

- Performance Reporting	11

- Non-Compliance	12

- Approval	12

# Introduction

  - Broadacres Housing Association (BHA) is a successful, innovative, not-for- profit housing association based in the market town of Northallerton in North Yorkshire. BHA owns and manages more than 6900 homes and has stock in Hambleton, Darlington, Richmondshire, Harrogate, Redcar, Cleveland, Ryedale, Scarborough, Selby, the East Ridling of Yorkshire, York and Leeds.

  - BHA provides a range of services to customers, all aimed at meeting the vision: ‘to be the best rural housing association in the country’. Whether it is carrying out repairs to ensure homes are kept to the highest standards or providing ways for customers to save money, BHA strives to ensure their customers receive a first class customer experience and ultimately that customers are proud to say they live in a BHA home.

  - BHA is regulated by the Regulator of Social Housing (RSH). The Directors Management Team (DMT) and Board set  and  oversee corporate strategy, including the approval of all compliance policy principles.

  - Breathing in air containing asbestos fibres can lead to asbestos-related diseases, mainly cancers of the lungs and chest lining. Asbestos is only a risk to health if asbestos fibres are released into the air and breathed in. Past exposure to asbestos currently kills around 4,500 people a year in Great Britain.

  - There is usually a long delay between first exposure to asbestos and the onset of the disease (15-60 years). Only by preventing or minimising these exposures now can asbestos-related disease eventually be reduced.

  - Any BHA home or building built or refurbished before the year 2000 may contain asbestos. As long as the asbestos containing material (ACM) is in good condition and is not going to be disturbed or damaged, there is negligible risk. However, if it is disturbed or damaged it can become a danger to health, because people may breathe in any asbestos fibres released into the air. Workers who carry out repairs and maintenance work are at particular risk. If asbestos is present and can readily be disturbed, is in poor condition and not managed properly, others who may be occupying the homes or buildings could also be put at risk.

  - BHA is responsible for the maintenance and repairs to homes, non-domestic areas and other buildings, many of which will have been constructed using asbestos containing materials. As such, the organisation has a legal ‘duty to manage’ asbestos in its homes and buildings and this must be done in accordance with the Control of Asbestos Regulations (CAR) 2012. BHA also acknowledges and accepts its responsibilities in accordance with the Health & Safety at Work Act 1974.

# Scope

  - BHA must establish a policy which meets the requirements of the Control of Asbestos Regulations (CAR) 2012, which came into force on 6th April 2012. In addition to this, the policy must provide assurance to BHA that measures are in place to identify, manage and/or mitigate risks associated with asbestos.

  - BHA must also ensure that compliance with asbestos is formally reported at DMT and Board level, including the details of any non-compliance and planned corrective actions.

  - The policy is relevant to all BHA employees, customers, contractors and other persons or other stakeholders who may work on, occupy, visit, or use its premises, or who may be affected by its activities or services.

  - It should be used by all to ensure they understand the obligations placed upon BHA to maintain a safe environment for customers and employees, within the home of each customer, and within all communal areas of buildings.

  - The policy is also relevant for maintaining a safe environment for all customers and employees within all BHA non-domestic properties, where the organisation has an obligation to do so.

# Regulatory Standards, Legislation and Codes of Practice

  - Regulatory Standards - the application of this policy will ensure compliance with the regulatory framework and all relevant consumer standards.

  - Legislation - the principle legislation applicable to this policy is the Control of Asbestos Regulations (CAR) 2012, which came into force on 6th April 2012. BHA has a legal obligation under Part 2, Section 4 of the legislation (Duty to manage asbestos in non-domestic properties) and is the ‘Duty Holder’ for the purposes of the legislation. BHA is the ‘duty holder’ by virtue of the fact that they own and manage 6,000 homes and buildings housing customers and leaseholders through the tenancy and lease agreement obligations it has.

  - Approved Codes of Practice and Guidance – the principle codes of practice and guidance applicable to this policy are:

    - ACoP L143 - ‘Managing and working with Asbestos’ (Second edition December 2013)

    - HSG264 - ‘Asbestos: The survey guide’ (Second edition 2012, this holds ACoP status)

    - HSG248 – ‘Asbestos: The analysts guide for sampling, analysis and clearance procedures’ (First edition 2006)

    - HSG247 - ‘Asbestos: The licensed contractors’ guide’ (First edition 2006)

    - HSG227 - ‘A comprehensive guide to managing asbestos in premises’ (First edition 2002)

    - HSG210 - ‘Asbestos Essentials – A task manual for building, maintenance and allied trades and non-licensed asbestos work’ (Third edition 2012)

  - Sanctions - BHA acknowledges and accepts its responsibilities in accordance with the regulatory standards, legislation, and codes of practice, and that failure to discharge these responsibilities properly could lead to a range of sanctions including prosecution by the Health & Safety Executive under the Health & Safety at Work Act 1974, prosecution under the Corporate Manslaughter and Corporate Homicide Act 2007 and via a serious detriment judgement from the Regulator of Social Housing (RSH).

  - Customers and Leaseholders - BHA will use the legal remedies available within the terms of the tenancy and lease agreement should any customer, leaseholder or shared owner refuse access to carry out essential asbestos related inspection and remediation works.

# Additional Legislation

  - This asbestos policy also operates in the context of the following additional legislation:

    - Health and Safety at Work Act 1974

    - The Management of Health and Safety at Work Regulations 1999

    - The Workplace (Health Safety & Welfare) Regulations 1992

    - Personal Protective Equipment at Work Regulations 1992

    - Hazardous Waste (England & Wales) Regulations 2005 (Amendment 2009)

    - Control of Substances Hazardous to Health (COSHH) Regulations (as amended) 2002

    - Construction, Design and Management Regulations 2015

    - Defective Premises Act 1972

    - Landlord and Customer Act 1985

    - Data Protection Act 1998

    - RIDDOR 2013

# Obligations

  - The duty to manage asbestos is included in the Control of Asbestos Regulations (CAR) 2012. The duty requires BHA to manage the risk from asbestos by:

    - Finding out if there is asbestos in the homes and buildings owned and managed by the organisation (or assessing if asbestos containing materials are liable to be present and making a presumption that materials contain asbestos, unless the  organisation has strong evidence that they do not), identifying its location and identifying what condition it is in. If the home or building was built prior to the year 2000 the organisation will assume asbestos is present. If the home or building was built after the year 2000 asbestos is unlikely to be present and no further action will be required.

    - Making and keeping an up-to-date record (referred to as the ‘asbestos register’) of the location and condition of the asbestos containing materials or presumed asbestos containing materials in non domestic buildings owned and managed by the organisation.

    - Assessing the risk from the asbestos containing materials found.

    - 5.1.4Preparing an Asbestos Management Plan that sets out in detail how the organisation  is  going  to  manage  the  risk  from  the  asbestos  containing

materials.

5.1.5Taking the steps needed to put the asbestos management plan into action. 5.1.6Reviewing	and	monitoring	the	asbestos	management	plan	and	the

arrangements made to put it in place; and

5.1.7Setting up a system for providing information on the location and condition of the asbestos containing materials to anyone who is liable to work on or disturb these materials.

  - Anyone who has information on the whereabouts of asbestos in homes and buildings is required to make this available to the organisation as the ‘duty holder’, but the organisation then have to assess its reliability.

  - Those who are not duty holders, but control access to the organisation’s homes or buildings, have to co-operate with the organisation in managing the asbestos present.

# Statement of Intent

  - BHA recognises that the main hazard in relation to asbestos is the non- identification of asbestos and asbestos containing materials and as such will protect those persons potentially exposed to asbestos as far as is reasonably practical by minimising the exposure using appropriate control measures and working methods.

  - BHA accepts that asbestos is likely to be present in the majority of its properties built prior to the year 2000 and will therefore manage these properties accordingly.

  - In order to fully comply with the legislation BHA will have a Board approved Asbestos Policy. In  addition  the organisation will  have an Asbestos Management Plan and an Asbestos Register which will hold records of the non domestic assets which have asbestos containing materials in them.

  - BHA will employ competent external contractors (UKAS accredited) to undertake asbestos management surveys. Licensed asbestos removal contractors will be employed to carry out remediation works (where necessary) to non-domestic (communal) and domestic properties as outlined in section 8 of this policy.

  - Domestic properties will include all void properties where repairs or maintenance work is taking place and is likely to disturb any asbestos containing materials within these properties. Domestic properties will also include any occupied properties where repairs or maintenance work is taking place and is likely to disturb any asbestos containing materials within these properties.

  - BHA will undertake an intrusive refurbishment and demolition (R&D) survey to domestic and non-domestic properties, prior to planned maintenance works taking place, to the areas within the property that are likely to be disturbed as part of the proposed works. They will also undertake a management survey to the remainder of the property as part of the same survey. This will be in accordance with the criteria set out in the BHA Asbestos Management Plan, which should be read in conjunction with this policy.

  - BHA considers good communication essential in the safe delivery of asbestos management and will therefore ensure that information about asbestos containing materials (known or suspect) is made available to every person liable to disturb it, accidentally or during the course of the work. This includes employees and customers.

  - BHA will ensure all employees, operatives and contractors have adequate asbestos survey information or training to enable them to manage and/or work safely with or around asbestos.

  - BHA will provide advice to customers regarding asbestos containing materials through leaflets and information held on the organisations website.

  - BHA will ensure that all visitors are made aware of the Asbestos Register and the emergency arrangements on site.

# Compliance Risk Assessment and Inspection Testing Programmes

  - Non-Domestic Stock – BHA will review existing asbestos management survey information prior to carrying out any repairs or planned maintenance works which may involve working on, or adjacent to, any asbestos containing materials within a non-domestic (communal) property. This is to ensure that any asbestos containing materials likely to pose a risk are identified prior to works commencing and the details passed onto the relevant operatives or external contractors and managed in an appropriate way. Where required a new hybrid R&D/management survey should be commissioned prior to work commencing.

  - BHA will ensure that all non-domestic (communal) and ‘other’ properties in ownership or management have an asbestos management survey carried out. All surveys will comply with the CAR 2012 legislation and therefore be dated after 6th April 2012 when the legislation came into effect.

  - Thereafter all non-domestic (communal) and ‘other’ properties will have a re-inspection survey and an assigned re-inspection date where applicable. This date will be in accordance with the specific needs of the building. It will either be annually or at a period dictated by the previous survey/re- inspection. Re-inspection dates may change following the re-categorisation of a property or a building.

  - BHA will not need to re-inspect any non-domestic (communal) or ‘other’ properties built after the year 2000, or where the initial asbestos management survey confirms that there are no asbestos containing materials detected.

  - Domestic Stock - BHA will review existing asbestos management survey information prior to carrying out any void or planned maintenance works which may involve working on or adjacent to any asbestos containing materials within an occupied domestic property. This is to ensure that any asbestos containing materials likely to pose a risk are identified prior to works commencing and the details passed onto the relevant contractors and managed in an appropriate way.

  - Where there is no previous asbestos related information in respect of a domestic property which requires void repairs, planned maintenance work or major repairs , a hybrid R&D/management survey will be undertaken and the scope of the survey agreed in accordance with the works due to be carried out.

  - Where a BHA operative identifies any potentially asbestos containing materials during a responsive repair visit the work will be suspended until the existing asbestos survey is reviewed or a  hybrid R&D/management survey is undertaken with the scope of the survey agreed in accordance with the works due to be carried out.

  - BHA will provide all BHA operatives existing asbestos information using EDRM.

  - BHA will not need to re-inspect any domestic properties built after the year 2000, or where the initial asbestos management survey confirms that there are no asbestos containing materials detected.

  - Refurbishment Work - BHA will undertake an intrusive refurbishment and demolition (R&D) survey to domestic and non-domestic properties prior to planned maintenance works taking place to the areas of the property that are likely to be disturbed as part of the proposed works. They will also undertake a management survey to the remainder of the property as part of the same R&D survey. This will be in accordance with the criteria set out in BHA’s Asbestos Management Plan, which should be read in conjunction with this policy.

# Compliance Follow up Work

  - BHA will ensure there is a robust process in place for the management of any follow-up works required following the completion of an asbestos management survey.

  - BHA will ensure that following asbestos surveys, any asbestos containing materials that are found to be in a poor condition or that are likely to be regularly/easily damaged or disturbed through the normal use of a dwelling will be repaired, protected, or removed at the earliest opportunity.

  - Where there is any doubt as to the composition of the material uncovered, a sample will be taken and analysed to facilitate the instruction of remedial works where required.

  - Where asbestos is positively identified and as a result of a risk assessment (conducted in accordance with published guidance) removal, sealing or encapsulation is recommended, this will be carried out for:

  - Non-licensed works – as defined in regulation 2 of the CAR 2012 – by specifically trained contractors (including operatives working for the in-house repairs team) with appropriate equipment and working procedures in place which are sufficient to comply with the CAR 2012.

  - Notifiable non-licensed works – as defined in regulation 2 of the CAR 2012 – by a licensed asbestos removal contractor (LARC) licensed by the Health & safety Executive in compliance with the CAR 2012, or

  - Licensed works - as defined in regulation 2 of the CAR 2012 – by a licensed asbestos removal contractor (LARC), licensed by the Health & safety Executive in compliance with the CAR 2012.

  - BHA will ensure there is a robust process in place to investigate and manage all RIDDOR notices issued with regard to asbestos.

  - Where an asbestos containing material has been removed in whole or in part, it will be replaced with a material that has no asbestos content and fulfils the equivalent function of the original asbestos material, e.g. for fire protection.

# Record Keeping

  - BHA will establish and maintain a programme of non-domestic surveys and re- inspections and an Asbestos Register(TEAMS), stored in QL, of all the asbestos containing materials by type, address, location and condition.

  - Aareon QL will be used to record the details of all asbestos surveys undertaken on BHA’s non-domestic (communal), domestic and other properties. This will include the date of the most recent survey inspection.

  - The remediation works identified and subsequently completed should be recorded in QL. Aareon QL will also record the date of any subsequent re-inspection required, where applicable.

  - BHA will hold  and maintain accurate records against every non-domestic (communal), domestic and ‘other’ property where applicable and all survey reports and other certification will be stored on the organisation’s shared drive and within the asbestos consultant’s web-based portal.

  - Appropriate asbestos information will be made available to all interested stakeholders as required.

# Roles and Responsibilities

  - BHA Board will have overall governance responsibility for ensuring the asbestos policy is fully implemented to ensure full compliance with the regulatory standards, legislation and codes of practice. As such the Board will formally approve this policy and review it every year (or sooner if there is a change in regulation, legislation or codes of practice).

  - The Board will receive regular updates on the implementation of the Asbestos Policy and asbestos performance along with notification of any non- compliance issue which is identified. This is so they have assurance that the policy is operating effectively in practice.

  - DMT will receive reports in respect of asbestos management performance and ensure compliance is being achieved. They will also be notified of any non-compliance issue identified.

  - The Customer Experience Director has strategic responsibility for the management of asbestos and ensuring compliance is achieved and maintained. The Compliance  Manager will oversee the implementation of the Asbestos Policy.

  - The Compliance Manager will be responsible for overseeing the delivery of the agreed survey inspection programmes and the prioritisation and implementation of any works arising from the surveys.

  - The housing management and supported housing teams will provide key support in gaining access into properties where access is proving difficult and use standards methods to do so. They will also facilitate the legal process to gain access as necessary.

# Competent Persons

  - BHA will ensure that the manager with lead responsibility for operational delivery is P405 qualified (as defined in 10.5).. If they do not have P405 or one of the additional qualifications already, they should obtain  them within 18 months of the approval of this policy.

  - BHA will ensure that competent (UKAS accredited) contractors are procured and appointed to deliver initial asbestos management surveys and the ongoing programme of surveys thereafter. The Compliance Manager will check the relevant qualifications of employees working for these contractors on an annual basis and evidenced appropriately.

  - BHA will ensure that competent licensed asbestos removal contractors (LARC) are procured and appointed for all notifiable non-licensed work or licensed works. The Compliance Manager will check the relevant qualifications of employees working for these contractors on an annual basis and evidenced appropriately.

# Training

  - This policy and the Asbestos Management Plan that supports it will be subject to a range of training across BHA and will involve all relevant stakeholders. The training will be bespoke to the individual stakeholders and refresher training will be provided as appropriate.

  - Training will include team briefings for those employees who need to have a basic understanding and awareness of asbestos but who may not be actively involved in the delivery of the asbestos policy, plan and programmes of surveys and works. This will be basic asbestos awareness training and will include delivery at relevant team meetings. BHA’s asbestos management consultancy should be able to provide this training.

  - On the job training will be provided to those employees who will be responsible for managing the programme of surveys and remediation works as part of their daily job. BHA’s asbestos management consultancy should be able to provide this training.

  - Regular tool box talks should be given by BHA’s contractor partners to their operatives in the form of appropriate asbestos awareness training and provide evidence of the same.

  - Monitors briefings will be provided to those employees involved in the monitoring of compliance performance in relation to asbestos.

# Performance Reporting

  - Robust key performance indicator (KPI) measures will be established and maintained to ensure BHA is able to report on performance in relation to asbestos.

  - KPI measures will be produced and provided at DMT level on a weekly basis and at Board level on a monthly basis. As a minimum these KPI measures will include reporting on:

The total number of:

- Non-domestic properties

- Non-domestic properties on  the  asbestos  management  survey/re- inspection programme

- Non-domestic properties with a valid ‘in date’ asbestos management survey and/or re-inspection

- Non-domestic properties where the survey and/or re-inspection has expired and is ‘out of date’

- % compliance

As well as an explanation of the:

- Current position

- Corrective action required

- Anticipated impact of corrective action

- Progress with completion of follow-up works.

- Quality assurance checks on contractors work.

  - external contractors will undertake5% quality assurance audit checks, as required by UKAS, carry out an independent audit of asbestos at least once every two years. This audit will specifically test for compliance with the regulation, legislation and codes of practice and identify any non-compliance issues for correction.

# Non-Compliance

  - Any non-compliance issue identified at an operational level will be formally reported to the Customer Experience Director in the first instance.

  - The Customer Services Director will agree an appropriate course of corrective action with the Compliance Manager in order to address the non-compliance issue and report details of the same to the DMT.

  - The DMT will ensure the Board are made aware of any non-compliance issue so they can consider the implications and take action as appropriate.

  - In cases of a serious non-compliance issue the DMT and Board will consider whether it is necessary to disclose the issue to the Regulator of Social Housing in the spirit of co-regulation as part of the Regulatory Framework.

# Approval

Strategic Lead:	Customer Experience Director 	 Sign/date:

Chair Board:

Sign/date: Board

Member:

Sign/date: